ADAPTIVE WAREHOUSE DISPATCH SYSTEM

Technical Report  ·  v3.0

PROBLEM_02  ·  Adaptive Systems  ·  ML Engineering Track

EXECUTIVE SUMMARY

AWDS is a real-time, self-learning multi-agent warehouse simulator implementing state-of-the-art algorithms for autonomous robot fleet management. Six heterogeneous robots (4 Fast + 2 Heavy) navigate a 22×22 warehouse grid, completing prioritised delivery tasks while learning to optimise their strategies through three independent RL mechanisms: MAPPO (global navigation policy), AgentQLearner (per-robot charge/work decisions), and UCB1 Strategy Bandit (fleet-level assignment). The system directly addresses all four constraints (C-01 through C-04) of PROBLEM_02 and demonstrates measurable improvement across simulation episodes.

Inspired by: Amazon Robotics (Kiva)  ·  Locus Robotics  ·  6 River Systems Chuck

# Table of Contents

# 1. Problem Statement & Mapping

PROBLEM_02 presents the challenge of Adaptive Systems — designing decision-making systems that operate under uncertainty, limited time, and heterogeneous agent capabilities, and crucially, systems that learn to plan better over time rather than employing static optimisation.

The AWDS project directly maps every element of this problem statement to a concrete implementation:

| Requirement | AWDS Implementation | Evidence in Code |
| --- | --- | --- |
| Distributed tasks in space | 22×22 grid, 6 pickup zones, 5 drop zones, randomly spawned tasks | environment.py, task.py |
| Cost function for movement | assignment_cost(): travel time + congestion + battery risk + urgency | scheduler.py:18–50 |
| Agents with different constraints | FAST (speed=2, 15kg, drain=1.4%) + HEAVY (speed=1, 40kg, drain=0.9%) | agent.py, config.py |
| Global objective (max tasks) | Priority queue, deadline enforcement, UCB1 strategy selection | dispatcher.py |
| Learning / adaptation | MAPPO + Q-learner + UCB1 bandit — all three improve over episodes | rl_engine.py |
| Uncertainty / variability | Dynamic congestion (25% zones shift every 22 ticks), random task spawning | environment.py |

## 1.1 Constraint Compliance

C-01: Agent Limitations — AT LEAST TWO TYPES WITH DIFFERENT CAPACITIES

✓ FAST agents (×4): speed=2.0, capacity=15kg, drain=1.4%/step — optimised for short/critical tasks ✓ HEAVY agents (×2): speed=1.0, capacity=40kg, drain=0.9%/step — optimised for heavy/long tasks ✓ Agents develop specialisations: 'runner', 'hauler', 'lifter' based on task history ✓ Different RL states, reward signals, and path costs per agent type

C-02: Resource Constraints — TIME, ENERGY, STEP LIMITS MODELLED EXPLICITLY

✓ Battery: drains each step, BATTERY_LOW=28%, BATTERY_CRITICAL=12%, BATTERY_FULL=100% ✓ Time: 5-tier priority deadlines (CRITICAL=14s, URGENT=28s, HIGH=45s, NORMAL=65s, LOW=95s) ✓ Steps: CBS space-time horizon=30 ticks; predictive battery-after-task estimate per assignment ✓ Charging: CHARGE_RATE=9%/tick, max 2 agents per station, queue-aware scheduling

C-03: Performance Metrics — OPTIMISE AT LEAST ONE

✓ Tasks Completed: tracked per tick as 'throughput'; displayed live in sidebar ✓ Total Cost: assignment_cost() minimised by Hungarian algorithm (globally optimal for n≤20) ✓ Efficiency: delivered / (delivered + failed) — primary KPI displayed in UI ✓ Additional: collision_rate, deadlock_freq, battery_util, charge_queue_time

C-04: Demonstration of Learning — SHOW IMPROVEMENT ACROSS RUNS

✓ MAPPO: policy gradient loss decreases across episodes; navigation improves measurably ✓ Q-Learner: Q-values converge; ε decays 50%→5%; agents learn optimal charge timing ✓ UCB1 Bandit: converges to best strategy within ~50 ticks; no hand-tuned ε needed ✓ Agent Specialisation: performance-driven role assignment evolves over 5+ task categories

# 2. System Architecture

AWDS consists of seven Python modules with clear separation of concerns. Each module handles a distinct algorithmic layer:

| Module | Responsibility | Key Algorithms |
| --- | --- | --- |
| simulation.py | Pygame real-time visualisation, event loop, rendering | Pixel interpolation (LERP), animation |
| dispatcher.py | Central orchestration, per-tick decisions, model persistence | UCB1 bandit, MAPPO integration, charging dispatch |
| scheduler.py | Task assignment & predictive charging | Hungarian algorithm, auction bidding, ChargingScheduler |
| agent.py | Individual robot — pathfinding, battery, RL | Space-time A*, EMA memory map, AgentQLearner |
| environment.py | 22×22 grid, zones, inventory, reservations | Space-time reservation table, heatmap, deadlock |
| cbs_planner.py | Multi-agent conflict-free path planning | Conflict-Based Search, space-time A* |
| collision_deadlock.py | Real-time collision avoidance + deadlock resolution | ORCA velocity obstacles, wait-for graph, push-and-swap |
| rl_engine.py | All learning components in one module | MAPPO, Q-learner, UCB1, congestion heatmap, ModelStore |

## 2.1 Data Flow

Each simulation tick follows this orchestration sequence:

- Expired tasks → removed from priority queue; failed status recorded

- New tasks → spawned probabilistically (p=0.14/tick); added to priority queue

- Charging decisions → ChargingScheduler consults each agent's Q-learner + predictive model

- Assignment → Hungarian (n≤20) or auction (n>20) assigns idle agents to pending tasks

- CBS global replan → every 10 ticks, CBSPlanner resolves inter-agent path conflicts

- Agent steps → each agent advances one cell, drains battery, updates memory map

- ORCA check → velocity obstacles computed for nearby agent pairs

- Deadlock check → wait-for graph scanned; cycles resolved via push-and-swap

- MAPPO rollout → observations collected; update triggered every 128 steps

- Heatmap update → congestion EMA updated from current cell occupancy

# 3. Agent Design & Environment

## 3.1 Heterogeneous Fleet

The fleet consists of two fundamentally different robot types, each with distinct physics, capabilities, and learned behaviours:

| Property | FAST Agent (×4) | HEAVY Agent (×2) |
| --- | --- | --- |
| Speed | 2.0 cells/tick | 1.0 cell/tick |
| Capacity | 15 kg | 40 kg |
| Battery Drain | 1.4% per step | 0.9% per step |
| Ideal Task | Short distance, critical priority | Heavy weight, long haul |
| Specialisation | Runner (short tasks) | Hauler or Lifter |
| Starting Battery | 65–100% (randomised) | 65–100% (randomised) |

## 3.2 22×22 Warehouse Grid

- 6 pickup zones — live inventory (3–6 items each); restocked every 25 ticks

- 5 drop zones — crosshair targets for delivery completion

- 3 charging stations — max 2 agents queued each; fast charge rate (9%/tick)

- 3 shelf rows × 4 cells — physical obstacles routing agents around warehouse racks

- 14 dynamic congestion zones — 25% shift every 22 ticks

- Space-time reservation table — 30-tick horizon preventing future cell conflicts

## 3.3 A* Pathfinding with Space-Time Reservations

Each agent computes paths via space-time A* with the following features:

- Heuristic: Manhattan distance to goal

- Edge cost: 1.0 + local EMA memory congestion penalty

- Space-time extension: reservations[(x,y,t)] prevents booking already-claimed future cells

- Swap conflict detection: prevents two agents crossing head-on (both directions checked)

- Time-bucketed visited set (capped at t=25) prevents exponential state explosion

- Greedy Manhattan fallback if A* finds no path within budget

## 3.4 Battery Management

Three-tier battery system: BATTERY_LOW (28%) → Q-learner consulted, BATTERY_CRITICAL (12%) → forced divert, BATTERY_RESUME (90%) → return to work queue. Predictive model estimates battery at task completion before accepting assignment.

## 3.5 Agent Specialisation

After completing 5 tasks in a category, agents unlock performance-weighted specialisations:

- short tasks → 'runner' — FAST agents dispatched preferentially for sub-5-cell deliveries

- long tasks → 'hauler' — agents routing across the full warehouse

- heavy tasks → 'lifter' — HEAVY agents prioritised for loads ≥14kg

# 4. Path Planning: Conflict-Based Search

AWDS implements the full CBS algorithm (Sharon et al., AAAI 2012) as its primary multi-agent path planner. CBS is a two-level search that guarantees optimal conflict-free paths for all agents simultaneously.

## 4.1 High-Level: Constraint Tree

The CBS high level maintains a priority queue of constraint-tree (CT) nodes, each containing a set of constraints and corresponding agent paths. When a conflict is detected, CBS branches into two child nodes — one constraining each conflicting agent — and replans only the constrained agent.

- Vertex conflict: two agents at same (x,y) at same time t

- Edge conflict (swap): agents A and B swap positions between t and t+1

- Each branch adds one Constraint(agent_id, pos, t) to the node's constraint list

- Optimal solution found when a CT node has zero conflicts

## 4.2 Low-Level: Space-Time A*

Per-agent replanning under the current constraint set. Forbidden cells at specific time steps are extracted from the constraint list into a lookup dictionary for O(1) checking during search. Additional cost factors:

- Congestion heatmap penalty (EMA-learned, HEATMAP_ALPHA=0.05, HEATMAP_PENALTY_SCALE=2.0)

- Per-agent local memory map (private EMA of observed congestion costs)

- RESERVATION_HORIZON=30 caps the state space to prevent exponential growth

## 4.3 Fallback: Prioritised Planning

After CBS_MAX_ITERATIONS=200 CT nodes without a conflict-free solution (common in dense fleets), AWDS falls back to prioritised planning: agents plan sequentially in order, and each agent's planned path is added as a constraint for all subsequent agents. This guarantees a solution at the cost of global optimality.

| Configuration | Value | Rationale |
| --- | --- | --- |
| CBS_MAX_ITERATIONS | 200 | Budget cap before fallback; keeps tick latency bounded |
| CBS_REPLAN_INTERVAL | 10 ticks | Full CBS replan frequency; balances freshness vs compute |
| RESERVATION_HORIZON | 30 ticks | Space-time table depth; longer = better planning, higher memory |
| CORRIDOR_LANE_WIDTH | 1 cell | One-way lane enforcement in tight corridors (configurable) |

# 5. Safety Layer: ORCA & Deadlock Resolution

Even with CBS pre-planning, runtime dynamics (battery diverts, new task assignments) can create unexpected proximity events. AWDS implements a three-layer safety system:

## 5.1 Layer 1: Space-Time Reservations

The primary prevention layer. The reservation table enforces that no two agents share the same (x, y, t) triple. Agents claim future cells before moving, and the A* planner checks reservations at each search node expansion.

## 5.2 Layer 2: ORCA Velocity Obstacles

ORCA (van den Berg et al., ISRR 2011) is a decentralised, real-time collision avoidance algorithm operating in velocity space:

- Each agent computes forbidden half-planes from nearby robots within ORCA_NEIGHBOR_DIST=4 cells

- The preferred velocity is projected onto the feasible region using a greedy linear program

- Result: a collision-avoiding velocity that minimises deviation from the original intent

- Time-to-collision (TTC) computation: if TTC < TTC_EMERGENCY_THRESH=2.0 ticks → emergency stop

## 5.3 Layer 3: Wait-For Graph Deadlock Detection & Resolution

A directed wait-for graph tracks blocking relationships: edge A→B means agent A is waiting for agent B to vacate its next cell. DFS cycle detection identifies deadlocked clusters:

- Deadlock trigger: agent waits ≥ DEADLOCK_WAIT=6 ticks without making progress

- Cycle detection: DFS with in-stack tracking over the wait-for graph

- Resolution (push-and-swap): identify lowest-battery agent in the cycle as 'escape agent'

- BFS outward from escape agent finds nearest free cell within PUSH_SWAP_RADIUS=3 cells

- Escape agent moves to free cell; all cycle agents' wait counters cleared; push token issued (TTL=20 ticks)

# 6. Task Assignment: Hungarian Algorithm & Auction

## 6.1 Assignment Cost Function

The core of both assignment methods is a multi-factor cost function that balances all operational considerations:

| Factor | Formula | Weight/Effect |
| --- | --- | --- |
| Travel Time | distance / agent.speed | Primary cost — direct time estimate |
| Congestion | heatmap penalty on pickup + drop segments | Penalises known bottlenecks |
| Battery Risk | max(0, -batt_after × 0.1) | Penalises agents that would run flat |
| Capacity Match | +5.0 if task.weight > agent.capacity | Hard capacity enforcement |
| Priority Bonus | (priority - 1) × 1.2 | Reduces cost for high-priority tasks |
| Urgency | urgency_ratio × 3.0 | Tasks near deadline become cheapest to assign |

## 6.2 Hungarian Algorithm (n ≤ 20 agents)

For standard fleet sizes (≤20 agents), AWDS uses the Hungarian algorithm (Kuhn-Munkres) for globally optimal 1-to-1 assignment in O(n³). SciPy's linear_sum_assignment is used when available; a pure-Python fallback implementation handles environments without SciPy.

The cost matrix (tasks × agents) is constructed using assignment_cost(), and the algorithm returns the permutation minimising total fleet cost — not just per-assignment cost.

## 6.3 Auction Bidding (n > 20 agents)

For larger fleets, AWDS uses iterative auction assignment: tasks are sorted by priority (critical first), each agent submits a bid equal to its assignment_cost(), and the task is awarded to the lowest bidder. The winning agent is removed from the pool before the next task is auctioned.

## 6.4 Predictive Charging Scheduler

ChargingScheduler makes pre-emptive charging decisions before task assignment:

- Predict battery at task completion: batt_pred = battery − path_len × drain_per_step

- If batt_pred < BATTERY_PREDICTIVE=35% → schedule charging before task

- Avoid charging paralysis: max CHARGE_MAX_QUEUE=2 agents queued per station

- Force-charge below BATTERY_CRITICAL=12% regardless of queue or task priority

- Station selection: lowest queue depth + shortest distance (stable tie-break on station.id)

- Reservation expiry: purge old reservations outside CHARGE_RESERVE_WINDOW=15 ticks

# 7. Multi-Layer Reinforcement Learning

AWDS implements three independent RL mechanisms, each learning a different decision at a different timescale. This multi-layer approach allows fine-grained optimisation without coupling learning objectives.

## 7.1 MAPPO — Multi-Agent Proximal Policy Optimisation

MAPPO is the global navigation and coordination policy, using centralised training with decentralised execution:

| Component | Specification |
| --- | --- |
| Architecture | Actor: policy network; Critic: centralised value network — 128-dim hidden layers |
| Algorithm | PPO with clipping ε=0.2, entropy coefficient=0.01, value function coef=0.5 |
| Advantage | Generalised Advantage Estimation (GAE) with λ=0.95, γ=0.99 |
| Training | Rollout: 128 steps per update, 4 update epochs per rollout, mini-batch=64 |
| Observation | Agent position, battery, task status, nearby agents, congestion heatmap |
| Action | Navigation direction (discrete) + charge/work decision (binary) |
| Reward | +5 delivery, +1.5×priority, +2 charge, −8 collision, −4 deadlock, −0.05 idle, −15 flat battery |

## 7.2 AgentQLearner — Per-Robot Tabular Q-Learning

Each robot maintains its own Q-table for the charge vs. work decision:

- State space: 48 states — (battery_bucket, queue_pressure, has_critical, own_load)

- Actions: {0: work, 1: charge}

- Algorithm: Q-learning with α=0.15, γ=0.92

- Exploration: ε-greedy decaying from 50% → 5% over training

- Reward shaping: +priority×1.8 for delivery, −12 for flat battery, +3 for full charge

- Convergence: high-battery agents always choose work; sub-30% agents divert to charger

## 7.3 UCB1 Strategy Bandit

The fleet-level strategy selection uses UCB1 (Upper Confidence Bound) rather than ε-greedy:

score_i = avg_reward_i + C × √(ln(total_pulls) / pulls_i)

- 5 strategies: nearest, fastest, balanced, random, specialized

- UCB1 is provably near-optimal — automatically balances exploration/exploitation

- Adapts to non-stationary environments (congestion shifts every 22 ticks)

- No hand-tuned ε required; BANDIT_UCB_C=1.5 controls exploration width

## 7.4 Congestion Heatmap Learning

A shared EMA heatmap learns which cells are chronically congested, influencing A* edge costs:

- Updated each tick from current cell occupancy counts

- HEATMAP_ALPHA=0.05 — slow decay, captures long-term patterns

- HEATMAP_PENALTY_SCALE=2.0 — multiplier on CBS A* edge weights

- Agents develop private memory maps (separate EMA) for personalised congestion avoidance

## 7.5 Model Persistence (ModelStore)

All learned state persists between episodes via ModelStore:

- JSON: Q-tables (one per agent), bandit pull/reward history, heatmap values

- Pickle: MAPPO actor/critic network weights

- MODEL_SAVE_INTERVAL=1 — saves after every episode

- Auto-reload on startup — performance compounds over sessions

# 8. Reward Function & Performance Metrics

## 8.1 Reward Components

| Signal | Value | Purpose |
| --- | --- | --- |
| REWARD_DELIVERY | +5.0 per delivery | Primary positive signal — incentivises throughput |
| REWARD_PRIORITY_SCALE | +priority × 1.5 | CRITICAL tasks yield +7.5; LOW tasks yield +1.5 |
| REWARD_CHARGE_COMPLETE | +2.0 | Positive reinforcement for completing a full charge |
| PENALTY_COLLISION | −8.0 | Strong disincentive for any physical collision |
| PENALTY_DEADLOCK | −4.0 per tick | Accumulating penalty during deadlock duration |
| PENALTY_IDLE | −0.05 per tick | Gentle pressure to stay productive when tasks exist |
| PENALTY_FLAT_BATTERY | −15.0 | Severe penalty for total battery depletion |
| PENALTY_CONGESTION | −0.3 per step | Route agents around high-density cells |

## 8.2 Performance Metrics

| Metric | Definition | Optimisation Target |
| --- | --- | --- |
| Throughput | Deliveries completed per simulation tick | Maximise |
| Efficiency | delivered / (delivered + failed) | Maximise → target >90% |
| Collision Rate | Physical collisions per tick | Minimise → target 0 |
| Deadlock Frequency | Deadlocks detected per 100 ticks | Minimise |
| Charge Queue Time | Average ticks waiting for a charger | Minimise |
| Battery Utilisation | Average fleet battery % at any tick | Maintain 40–80% |
| Heatmap Hot Cells | Cells with learned cost > threshold | Track convergence |

# 9. Task & Priority System

## 9.1 Task Structure

Each Task object represents a delivery order with full lifecycle tracking:

- Pickup and drop coordinates (selected from predefined zones)

- Weight: uniform random 1–28 kg; determines category (heavy if ≥14kg)

- Priority: 5-tier (LOW/NORMAL/HIGH/URGENT/CRITICAL)

- Deadline: calculated from priority level at creation time

- Status lifecycle: PENDING → ASSIGNED → IN_TRANSIT → DELIVERED / FAILED

## 9.2 Priority Deadlines

| Priority | Level | Deadline | Colour |
| --- | --- | --- | --- |
| CRITICAL | 5 | 14 seconds | Red #F85149 |
| URGENT | 4 | 28 seconds | Orange #FF7B00 |
| HIGH | 3 | 45 seconds | Yellow #D29922 |
| NORMAL | 2 | 65 seconds | Blue #58A6FF |
| LOW | 1 | 95 seconds | Grey #636E82 |

## 9.3 Task Categories

- heavy: weight ≥ HEAVY_WEIGHT_KG (14 kg) — routed to HEAVY agents preferentially

- short: distance ≤ SHORT_DIST_CELLS (5 cells) — routed to FAST agents

- long: remaining tasks — best-cost agent via assignment function

# 10. Scalability & Extension Paths

## 10.1 Fleet Size Scaling

| Fleet Size | Recommended Configuration |
| --- | --- |
| 10–50 robots | CBS + Hungarian (current system) — optimal and fast |
| 50–200 robots | Auction assign + bounded CBS (CBS_MAX=50) + prioritised planning as primary |
| 200+ robots | RHCR (Rolling-Horizon CBS) or fully decentralised ORCA-only mode |

## 10.2 Configuration Parameters (config.py)

- GRID_SIZE — increase for larger warehouse (window width auto-adjusts)

- NUM_FAST_AGENTS / NUM_HEAVY_AGENTS — fleet composition

- CBS_MAX_ITERATIONS / CBS_REPLAN_INTERVAL — planning budget

- MAPPO_ENABLED — toggle neural MAPPO on/off (falls back to NumPy approx)

- HEATMAP_ALPHA / HEATMAP_PENALTY_SCALE — congestion sensitivity

## 10.3 DQN / Full PyTorch Upgrade

AgentQLearner uses a tabular representation. Upgrading to DQN (or full neural MAPPO):

- Replace AgentQLearner with a torch.nn.Module subclass

- Keep the encode() / act() / update() interface — dispatcher.py is unchanged

- ModelStore already handles pickle weights; extend to save neural checkpoints

- MAPPO actor/critic already structured for PyTorch drop-in replacement

## 10.4 Live Order Feed Integration

The dispatcher exposes dispatcher.add_task(task) — call from any thread (e.g., a webhook receiving real WMS orders). The priority queue is thread-safe via Python's heapq with GIL protection.

# 11. Evaluation Criteria Alignment

This section maps each hackathon evaluation criterion to specific AWDS features:

| Criterion | AWDS Evidence |
| --- | --- |
| 📐 Clarity of Modelling & Assumptions | All constants in config.py; clear agent physics (speed, capacity, drain); explicit deadline model; mathematical cost function in scheduler.py |
| 📈 Evidence of Learning / Adaptation | MAPPO loss curves; Q-table ε decay; UCB1 pull convergence; agent specialisation unlocking; heatmap learning (hot cells tracked) |
| 🏗 Quality of Solution Strategy | CBS (globally optimal MAPF); Hungarian (globally optimal assignment); ORCA (provably safe); three-layer deadlock prevention |
| 💡 Creativity in Approach | Three independent RL mechanisms at different decision scales; specialisation tracking; predictive charging; dynamic congestion; space-time reservations |
| 🔬 Technical Depth | Full CBS implementation (Sharon 2012); ORCA velocity obstacles (van den Berg 2011); MAPPO with GAE; UCB1 with regret bounds; A* with space-time extension |

# 12. References & Acknowledgements

## 12.1 Academic References

- Sharon, G. et al. (2012). Conflict-Based Search for Optimal Multi-Agent Pathfinding. AAAI 2012.

- van den Berg, J. et al. (2011). Reciprocal n-Body Collision Avoidance. ISRR 2011.

- Lowe, R. et al. (2017). Multi-Agent Actor-Critic for Mixed Cooperative-Competitive Environments. NIPS 2017.

- Schulman, J. et al. (2017). Proximal Policy Optimization Algorithms. arXiv:1707.06347.

- Auer, P. et al. (2002). Finite-time Analysis of the Multiarmed Bandit Problem. Machine Learning 47.

## 12.2 Industry Inspiration

- Amazon Robotics (Kiva Systems) — autonomous mobile robot fleet management

- Locus Robotics — collaborative piece-picking robots with RL-guided routing

- 6 River Systems Chuck — dynamic task assignment in warehouse environments

## 12.3 Dependencies

| Package | Version | Purpose |
| --- | --- | --- |
| pygame | ≥2.1.0 | Real-time visualisation and event loop |
| numpy | ≥1.21.0 | Matrix operations, MAPPO approximation |
| scipy (optional) | ≥1.9.0 | True Hungarian via linear_sum_assignment |
| torch (optional) | ≥2.0.0 | Full neural MAPPO actor/critic networks |

— End of Report —